# Sense HAT: First steps

If you are using this worksheet with a Raspberry Pi computer and a physical Sense HAT, then use the Mu development environment.

For the emulator use Trinket (ncce.io/ks4-senseHAT-emulator).

# Task .Getting started

Step 1

Copy the start code below into your development environment. This imports the Sense HAT library. You will need these two lines of code whenever you wish to use a Sense HAT.

| 1 2 | from sense_hat import SenseHat sense = SenseHat() |
| --- | --- |

Step 2

Now copy the following code into line 3.

This piece of code is a function that takes a message, formats it so that it can be shown on LED display, and scrolls it smoothly across.

| 3 | sense.show_message("Hello world!") |
| --- | --- |

Step 3

Save and run your program. You should see the words “Hello world!” scroll across the LED matrix.

Important: Do not save your file as sense_hat.py as it won’t work because it uses the same name as the module used for the Sense HAT.

# Task .Using colour

Step 1

The show_message function can take more than one argument. You can also specify the text_colour, back_colour, and the scroll_speed.

Copy the code below to see what happens:

sense.show_message("Hello world!", text_colour = (0, 0, 255))

Step 2

The scroll text should have appeared in blue. It appears in blue due to the three values that have been used between the brackets. The first value is for Red, the second is for Green, and the third is for Blue.

# R, G, B sense.show_message("Hello world!", text_colour = (0, 0, 255))

Modify the code so that the text appears in red instead of blue.

Step 3

For each colour you can use a value from 0–255. 0 is no colour, and 255 is full brightness.

Investigate the different colour options on this website and try a different colour with your scroll message:

https://www.w3schools.com/colors/colors_picker.asp

Step 4

Instead of entering the values each time, you can hold the colour values in variables.

red = (255, 0, 0) green = (0, 255, 0) blue = (0, 0, 255)

Create a variable with your chosen colour and use the variable name inside the brackets instead of the values.

Make sure that you create the variable before the show_message code.

Step 5

To change the back_colour you need to add another argument.

back_colour = blue

Modify your code so that it changes the background colour of the message as well.

# Explorer task .

Investigate the purpose of this code snippet:

sense.set_pixel(1, 5, red)

See if you can use it to draw a simple picture.

Note: The pixels will remain on the LED matrix until they are cleared. If you need to clear the screen, then you can use sense.clear()

Resources are updated regularly — the latest version is available at: ncce.io/tcc.

This resource is licensed under the Open Government Licence, version 3. For more information on this licence, see ncce.io/ogl.