# Setting up a Sense HAT

These instructions will guide you through setting up your Sense HAT with your Raspberry Pi computer.

# You will need:   .

Hardware

- A Raspberry Pi computer (Raspberry Pi setup instructions)

- A Sense HAT

Software

- The latest version of Raspberry Pi OS

- Python 3 (included with Raspberry Pi OS)

- Sense HAT for Python 3 (included with Raspberry Pi OS)

# Task   Power down your device

| Steps | Instructions |
| --- | --- |
| - Make sure that your Raspberry Pi computer is switched off. | ▷ Shutdown… ▷ Shutdown |
| - Unplug your Raspberry Pi computer. | Make sure that the power cable for the Raspberry Pi is unplugged. There should be no power going into your Raspberry Pi. |

# Task  Attach the Sense HAT

| Steps | Instructions |
| --- | --- |
| - Carefully align the pins. |  |
| - Gently press down. |  |

# Task  Power up your device

| Steps | Instructions |
| --- | --- |
| - Attach the power cable. | Attach the power cable and all other required cables (monitor, keyboard, mouse). |
| - Switch on the power. | As your Raspberry Pi boots up, the Sense HAT should display a rainbow of colours on the LED matrix. The lights should last for 15 to 20 seconds before switching off. If your Sense HAT does not display a rainbow or the rainbow does not disappear, power down again and check your connections before switching the Raspberry Pi back on. |

# Task  Testing your Sense HAT

| Steps | Instructions |
| --- | --- |
| - Open Mu. | ▷ Programming ▷ Mu |
| - Import the Sense HAT library. | Enter the following code to import the library. from sense_hat import SenseHat sense = SenseHat() |
| - Show a message to test your Sense HAT. | Enter the following code after the two previous lines. sense.show_message("Hello world!") |
| - Save your program. | Save the file as test.py or something similar. Note: You must never save Sense HAT files as sense_hat.py It will not work because other programs in the same folder will not be able to access the sense_hat module. |
| - Run your program. | Click Run and watch your message scroll across the LED matrix. If you receive the message ‘ModuleNotFoundError: No module named 'sense_hat'’ make sure that you have the Sense HAT module installed by going to the command line and typing: sudo apt-get install sense-hat |

Resources are updated regularly — the latest version is available at: ncce.io/tcc.

This resource is licensed under the Open Government Licence, version 3. For more information on this licence, see ncce.io/ogl.